# CompLaB3D Technical Tutorial

Comprehensive Guide to Biotic/Abiotic Kinetics, Cellular Automata, and Equilibrium Solver

# Table of Contents

1. Introduction

2. Biotic Kinetics (Monod Model)

3. Abiotic Kinetics

4. Cellular Automata (CA) Biofilm Expansion

5. Equilibrium Solver (Anderson Acceleration)

6. XML Configuration Guide

7. Test Cases and Validation

# 1. Introduction

CompLaB3D is a 3D reactive transport simulation framework using the Lattice Boltzmann Method (LBM). It supports both biotic (with microorganisms) and abiotic (without microorganisms) simulations.

## 1.1 Simulation Modes

Biotic Mode (biotic_mode = true):

- Simulates microorganism growth and activity

- Uses Monod kinetics for substrate consumption

- Biofilm expansion via Cellular Automata

- Biomass grows, decays, and spreads

Abiotic Mode (biotic_mode = false):

- No microorganisms present

- Pure transport (diffusion/advection) OR

- Transport + chemical reactions (abiotic kinetics)

- Useful for geochemical modeling

# 2. Biotic Kinetics (Monod Model)

When microorganisms are present (biotic_mode = true), substrate consumption and biomass growth follow Monod kinetics. This is the standard model for microbial growth.

## 2.1 The Monod Equation

The specific growth rate is calculated as:

mu = mu_max * C / (Ks + C)

Where:

- mu = specific growth rate (1/s)

- mu_max = maximum specific growth rate (1/s)

- C = substrate concentration (kg/m3)

- Ks = half-saturation constant (kg/m3)

## 2.2 Biomass Change

The biomass changes according to:

dB/dt = (mu - k_decay) * B

Where:

- B = biomass concentration (kg/m3)

- k_decay = decay rate constant (1/s)

## 2.3 Substrate Consumption

Substrate is consumed proportionally to biomass growth:

dC/dt = -(1/Y) * mu * B

Where:

- Y = yield coefficient (kg biomass / kg substrate)

## 2.4 Code Implementation

In defineKinetics.hh, the Monod kinetics are implemented:

// Monod kinetics calculation
T mu = mumax[iM] * C[iS] / (Ks[iM] + C[iS]);

// Biomass change
dB = (mu - k_decay[iM]) * B[iM] * dt;

// Substrate consumption
dC = -(1.0/Y[iM]) * mu * B[iM] * dt;

## 2.5 XML Configuration for Biotic Mode

<microbe>
    <substrate>substrate_A</substrate>
    <mumax>1.0e-5</mumax>      <!-- Max growth rate (1/s) -->
    <Ks>0.001</Ks>              <!-- Half-saturation (kg/m3) -->
    <Y>0.5</Y>                  <!-- Yield coefficient -->
    <kdecay>1.0e-6</kdecay>     <!-- Decay rate (1/s) -->
    <b0_biofilm>0.1</b0_biofilm><!-- Initial biomass (kg/m3) -->
</microbe>

# 3. Abiotic Kinetics

When biotic_mode = false and enable_abiotic_kinetics = true, chemical reactions occur without microorganisms. This is useful for modeling purely chemical systems.

## 3.1 Reaction Types

### 3.1.1 First-Order Decay

Reaction: A -> products

dC_A/dt = -k * C_A

Analytical solution: C_A(t) = C_A(0) * exp(-k*t)

// In defineAbioticKinetics.hh:
T k = 0.001;  // decay rate (1/s)
dC[0] = -k * C[0] * dt;  // substrate A

### 3.1.2 Bimolecular Reaction

Reaction: A + B -> C

rate = k * C_A * C_B

dC_A/dt = dC_B/dt = -rate

dC_C/dt = +rate

// In defineAbioticKinetics.hh:
T k = 100.0;  // rate constant (m3/kg/s)
T rate = k * C[0] * C[1] * dt;
dC[0] = -rate;  // A consumed
dC[1] = -rate;  // B consumed
dC[2] = +rate;  // C produced

### 3.1.3 Reversible Reaction

Reaction: A <-> B

net_rate = k_forward * C_A - k_reverse * C_B

At equilibrium: K_eq = k_forward / k_reverse = C_B / C_A

// In defineAbioticKinetics.hh:
T k_f = 0.002;   // forward rate
T k_r = 0.001;   // reverse rate (K_eq = 2)
T net_rate = (k_f * C[0] - k_r * C[1]) * dt;
dC[0] = -net_rate;  // A
dC[1] = +net_rate;  // B

### 3.1.4 Sequential Decay Chain

Reaction: A -> B -> C

// In defineAbioticKinetics.hh:
T k1 = 0.002;  // A -> B rate
T k2 = 0.001;  // B -> C rate
dC[0] = -k1 * C[0] * dt;           // A decays
dC[1] = (k1*C[0] - k2*C[1]) * dt;  // B: formed and decays
dC[2] = k2 * C[1] * dt;            // C accumulates

## 3.2 XML Configuration for Abiotic Mode

<options>
    <biotic_mode>false</biotic_mode>
    <enable_kinetics>true</enable_kinetics>
    <enable_abiotic_kinetics>true</enable_abiotic_kinetics>
</options>

<!-- No <microbe> sections needed -->
<!-- Define reactions in defineAbioticKinetics.hh -->

# 4. Cellular Automata (CA) Biofilm Expansion

When biomass exceeds a threshold, it spreads to neighboring cells using Cellular Automata rules. CompLaB3D implements two methods: "fraction" and "half".

## 4.1 When Does Expansion Occur?

Expansion is triggered when:

B_cell > B_max (maximum biomass capacity)

## 4.2 The "fraction" Method

Excess biomass is distributed equally to all valid neighbors:

1. Calculate excess: excess = B_cell - B_max

2. Find valid neighbors (pore or existing biofilm)

3. Distribute: each neighbor gets excess / N_neighbors

4. Parent cell keeps B_max

// In complab3d_processors_part2.hh:
T excess = B_cell - B_max;
int N = valid_neighbors.size();
T share = excess / N;
for (each neighbor) {
    B_neighbor += share;
}
B_cell = B_max;

## 4.3 The "half" Method

Parent cell splits, giving half to ONE random neighbor:

1. Calculate excess: excess = B_cell - B_max

2. Find valid neighbors

3. Select ONE random neighbor

4. Transfer: neighbor gets B_cell / 2

5. Parent keeps B_cell / 2

// In complab3d_processors_part2.hh:
T excess = B_cell - B_max;
int random_idx = rand() % valid_neighbors.size();
neighbor = valid_neighbors[random_idx];
B_neighbor += B_cell / 2;
B_cell = B_cell / 2;

## 4.4 XML Configuration

<microbe>
    <bmax>100.0</bmax>           <!-- Threshold for expansion -->
    <ca_expansion>fraction</ca_expansion>  <!-- or "half" -->
</microbe>

## 4.5 Comparison of Methods

| Aspect | fraction | half |
| --- | --- | --- |
| Distribution | Equal to all neighbors | All to one neighbor |
| Growth pattern | Smooth, uniform | Irregular, branching |
| Use case | Dense biofilms | Finger-like structures |

# 5. Equilibrium Solver (Anderson Acceleration)

CompLaB3D uses the Picard iteration with Anderson Acceleration to solve for steady-state concentration fields. This is faster than simple iteration.

## 5.1 The Problem

We want to find the steady-state solution where:

dC/dt = 0  (concentrations stop changing)

## 5.2 Picard Iteration (Fixed-Point)

Basic approach - iterate until convergence:

C_new = G(C_old)  // Apply one LBM step
repeat until ||C_new - C_old|| < tolerance

Problem: Simple Picard iteration can be SLOW (thousands of iterations).

## 5.3 Anderson Acceleration

Anderson Acceleration speeds up convergence by using information from previous iterations to make better guesses. It combines the last m iterations optimally.

Key idea:

C_new = sum(alpha_i * G(C_i))  where sum(alpha_i) = 1

The weights alpha_i are chosen to minimize the residual.

## 5.4 Algorithm Steps

1. Store history of last m iterations

2. Build matrix F of residuals: F_k = G(C_k) - C_k

3. Solve least-squares problem for optimal weights

4. Compute accelerated update

5. Apply relaxation: C = beta * C_accelerated + (1-beta) * C_old

## 5.5 Code Implementation

// In complab3d_equilibrium.hh:

// Store iteration history
std::vector<MultiScalarField3D<T>*> X_history;  // solutions
std::vector<MultiScalarField3D<T>*> F_history;  // residuals

// Anderson mixing
for (int k = 0; k < max_iter; k++) {
    // 1. Compute G(x_k) - one LBM step
    G_k = applyLBMStep(X_k);

    // 2. Compute residual
    F_k = G_k - X_k;

    // 3. Check convergence
    if (norm(F_k) < tolerance) break;

    // 4. Anderson acceleration (if enough history)
    if (k >= m) {
        // Build matrix and solve least-squares
        alpha = solveLeastSquares(F_history);

        // Accelerated update
        X_new = sum(alpha[i] * G_history[i]);
    } else {
        X_new = G_k;  // Simple Picard
    }

    // 5. Relaxation
    X_k = beta * X_new + (1-beta) * X_k;
}

## 5.6 XML Configuration

<options>
    <solver_type>equilibrium</solver_type>
</options>

<equilibrium>
    <tolerance>1e-6</tolerance>     <!-- Convergence criterion -->
    <max_iterations>10000</max_iterations>
    <anderson_depth>5</anderson_depth>  <!-- History size m -->
    <relaxation>0.5</relaxation>    <!-- Mixing parameter beta -->
</equilibrium>

## 5.7 Convergence Comparison

| Method | Typical Iterations | Convergence |
| --- | --- | --- |
| Simple Picard | 5000-50000 | Linear (slow) |
| Anderson (m=5) | 500-2000 | Superlinear (fast) |

# 6. XML Configuration Guide

## 6.1 Complete Biotic Example

<?xml version="1.0" ?>
<complab>
    <options>
        <biotic_mode>true</biotic_mode>
        <enable_kinetics>true</enable_kinetics>
    </options>

    <geometry>
        <file>geometry.dat</file>
        <nx>100</nx>
        <ny>50</ny>
        <nz>50</nz>
    </geometry>

    <simulation>
        <dt>0.1</dt>
        <total_time>86400</total_time>  <!-- 1 day -->
        <output_interval>3600</output_interval>
    </simulation>

    <substrate name="oxygen">
        <diffusion>
            <in_water>2.1e-9</in_water>
            <in_biofilm>1.0e-9</in_biofilm>
        </diffusion>
        <initial_concentration>0.008</initial_concentration>
        <boundary>
            <left type="dirichlet">0.008</left>
            <right type="neumann">0.0</right>
        </boundary>
    </substrate>

    <microbe name="aerobic_bacteria">
        <substrate>oxygen</substrate>
        <mumax>1.0e-5</mumax>
        <Ks>0.001</Ks>
        <Y>0.5</Y>
        <kdecay>1.0e-6</kdecay>
        <b0_biofilm>0.1</b0_biofilm>
        <bmax>100.0</bmax>
        <ca_expansion>fraction</ca_expansion>
    </microbe>
</complab>

## 6.2 Complete Abiotic Example

<?xml version="1.0" ?>
<complab>
    <options>
        <biotic_mode>false</biotic_mode>
        <enable_kinetics>true</enable_kinetics>
        <enable_abiotic_kinetics>true</enable_abiotic_kinetics>
    </options>

    <geometry>
        <file>geometry.dat</file>
        <nx>100</nx>
        <ny>30</ny>
        <nz>30</nz>
    </geometry>

    <simulation>
        <dt>0.1</dt>
        <total_time>10000</total_time>
        <output_interval>1000</output_interval>
    </simulation>

    <!-- Reactant A -->
    <substrate name="species_A">
        <diffusion>
            <in_water>1.0e-9</in_water>
            <in_biofilm>1.0e-9</in_biofilm>
        </diffusion>
        <initial_concentration>1.0</initial_concentration>
        <boundary>
            <left type="dirichlet">1.0</left>
            <right type="neumann">0.0</right>
        </boundary>
    </substrate>

    <!-- Product B (from A decay) -->
    <substrate name="species_B">
        <diffusion>
            <in_water>1.0e-9</in_water>
            <in_biofilm>1.0e-9</in_biofilm>
        </diffusion>
        <initial_concentration>0.0</initial_concentration>
        <boundary>
            <left type="dirichlet">0.0</left>
            <right type="neumann">0.0</right>
        </boundary>
    </substrate>

    <!-- NO <microbe> sections in abiotic mode -->
    <!-- Reactions defined in defineAbioticKinetics.hh -->
</complab>

# 7. Test Cases and Validation

## 7.1 Abiotic Test Cases

### Test 1: Pure Diffusion

Purpose: Verify diffusion without reactions

Setup: Species A diffuses from left (C=1.0) to right

Expected: Linear concentration profile at steady state

### Test 2: First-Order Decay (A -> products)

Purpose: Verify first-order reaction kinetics

Rate law: dC/dt = -k*C with k = 0.001 1/s

Validation: C(t) = C0 * exp(-k*t)

At t=1000s: C should be ~0.368 * C0

### Test 3: Bimolecular (A + B -> C)

Purpose: Verify second-order reaction kinetics

Rate law: rate = k * C_A * C_B

Validation: Mass balance A + B + C = constant

### Test 4: Reversible (A <-> B)

Purpose: Verify equilibrium is reached

Rate law: net = k_f*A - k_r*B

Validation: At equilibrium, B/A = k_f/k_r = K_eq

### Test 5: Decay Chain (A -> B -> C)

Purpose: Verify sequential reactions

Validation: B shows characteristic rise-then-fall; C accumulates

## 7.2 Running Test Cases

# Navigate to test directory
cd test_cases/abiotic

# Copy test files
cp test2_first_order_decay.xml CompLaB.xml
cp defineAbioticKinetics_test2.hh ../../src/defineAbioticKinetics.hh

# Recompile and run
cd ../../src
make clean && make
cd ..
./complab

# Check results in output/

## 7.3 Validation Criteria

| Test | Check | Tolerance |
| --- | --- | --- |
| Pure Diffusion | Linear profile | < 1% |
| First-Order | C(t)/C0 = exp(-kt) | < 5% |
| Bimolecular | Mass conserved | < 1% |
| Reversible | B/A = K_eq | < 5% |
| Decay Chain | A+B+C = constant | < 1% |